IFO: a  Program for Image-Reconstruction-Type Calculation of Atomic Distribution Functions for Disordered Materials

Authors: Valeri Petkov and Radostin Danev,

Department of Solid State Physics, Sofia University, Bulgaria

For contacts:  Petkov@phys.uni-sofia.bg or Petkov@pa.msu.edu

- Theoretical background:

The structure of disordered materials is commonly described in terms of radial distribution functions (RDFs) which give the probability of finding an atom at a distance r from a reference atom. RDFs are usually obtained by diffraction experiments employing X-rays, neutron or electrons. The relation between the reduced RDF, G(r), and the structure factor, S(q), which is the structure-dependant part of the diffraction spectrum recorded, is as follows:

G(r)  = (2/p) 0¥q[S(q)−1]sin(qr)dq.

With IFO G(r) is not calculated by directly applying  the Fourier transformation given above. Instead, G(r) is calculated by minimazing the functional, Q,

Q = lc2 + mI +  g½S1theor.- S1calc.½+ d½S2theor. -S2calc½,

where

c2 = k(S(q)exp. - S(q)comp.)2,

I = k=1N[−G(rk−2)+16G(rk−1)−30G(rk)+16G(rk+1)−G(rk+2)]2epowSG(rk)−G(rk−2)+epowSG(rk+2)−G(rk),

S1theor. =  q=0qmaxqi[S(qi)−1]Dq=2p2ρo ,

S2theor =r=0rmaxriG(ri)Dr=-1

and l, m, g, d are Lagrange-type multipliers. The minimization is carried out through a Monte Carlo procedure. For more information see :

- Petkov and R. Danev, J. Appl. Cryst. 31 (1998) 609-619.

To run IFO two files should be prepared in advance:

- a data file, containing the experimental structure factor data

- a command file, containing parameters guiding the Monte Carlo-type  procedure for calculating G(r)

- 2. Structure of the data file:

The file with experimental S(q) data have two columns of numbers in

free format- equidistant Dq steps and S(q) values, respectively.

WARNING: IFO CAN HANDLE UP TO 10,000 data points.

- 3. Structure of the command file: (for example see sn.ini; sin.ini; si.ini)

Line one: the name of the file with the experimental structure factor, S(q),

data

Line two: the name of the file with G(r) data currently calculated by IFO

Line three: the sampling interval in real space, Dr

Line four: the maximal real space distance to which G(r) is

calculated, rmax

Line five (lambda): Value of the respective Lagrange multiplier, l,

Line six (mju): Value of the respective Lagrange multiplier, m,

Line seven (gamma):  Value of the respective Lagrange multiplier, g,

Line eighth (delta): Value of the respective Lagrange multiplier, d,

Line nine: (kappa): a constant determining the percentage of Monte Carlo

steps to be accepted

Line ten: (NMCS) the total number of Monte Carlo steps to be completed

Line eleven: initial amplitude of the random change of a given

G(r) data point

Line twelve: (powR) - provides some flexibility with the application of

the noise function, I. If,

powR = -1, the noise in the low-r part of G(r) is reduced preferentially

powR = 1, the noise in the high-r part of G(r) is reduced preferentially

powR = 0, (recommended) , no preferences

Line thirteen: (powS) value of the exponential pre-factor, (see the bottom

line of the noise function, I, given above). powS = 1 or 2 is

recommended

Line fourteen: flag for switching on/off the constraint requiring that G(r)

behaves as -4prro at small values of r. When flag = 0 the

constraint is off. When flag = 1 - on.

Line fifteen: (rdown) the minimal r space distance to which the above

constraint is applied, if switched on.

Line sixteen: ( ro ) average atomic number density of the material in

(atoms/3).

--------------------------------------------------------------------------------------

The command file can be redacted by any text editor.

Important ! While editing, keep the file structure, including the blank lines and comments, intact. Simply replace the old names/values by new ones.

4. System requirements:

Operating system: DOS 5.0 or higher/MS Windows 95,98, NT

Hardware: PC386 or higher; minimum 2Mbytes of memory;

VGA/SVGA graphics card

- 5. Getting started:

- make a directory with a name of your choice. Copy the files

ifo.exe and dos4gw.exe in it.

- with the use of some text editor create/redact one xxx.ini file according the problem you explore.

- Supply/create a file with the experimental S(q) data and do not

- forget to put its name in the xxx.ini file

- iv) In the DOS command field/line -

Type ifo xxx.ini  and press  Enter .

6. Running IFO:

When IFO runs the following information appears on screen:

- the names of the files with experimental S(q) and current G(r) data

- rmax, Dr, the number of sampling points in real space and the current

amplitude, dG, of the random change in the G(r) data points

- the experimental qmax, Dq and the number of sampling points in reciprocal space,

- the number of Monte Carlo steps completed (should increase with the progress of the calculations);

the current percentage of accepted Monte Carlo steps (usually fluctuates); and the current values of :

c2 (should decrease),

the noise function I (should decrease),

the probability of accepting a given change (may fluctuate but usually decreases);

ro calculated according the S1 “sum-rule” (should approach the experimental value)

S2 “sum-rule” (should approach -1).

- v) by pressing  I  the following graphics appear:

the experimental S(q) data - blue

the reconstructed S(q) data (current) - white

the current changes in S(q) - red

the current difference S(q)exp- S(q)calc - green (in the lower part of

the screen)

To resume the calculations - press any key

- by pressing  G  the following graphics appear:

the current noise in G(r) - brown (upper part of the screen)

the starting G(r) data - blue

the calculated G(r) data (current) - white

the current changes in G(r) - red

the current difference G(r)start- G(r)calc - green (in the lower part of

the screen)

To resume the calculations - press any key

The program stops when the pre-set number of Monte Carlo steps is completed or  Q is pressed.

- 7. Results from running IFO:

When the programs stops two new data files are created:

Final.iff - q steps; reconstructed S(q) values; difference S(q)exp-

S(q)calc, (three columns in free format, respectively)

Final.gr - r steps; calculated G(r) values; difference G(r)start-

G(r)calc and calculated g(r)calc values (four columns in free

format, respectively)

If one would like to, could stop/resume the calculations several

times provided the file with current G(r)calc values has not been already

deleted manually.

_ A new run with the same data may be started by simply deleting the file

mentioned above manually.

- Test examples -  All these can be run by typing IFO xxx.ini & Enter.

- Percus-Yevick hard-sphere liquid structure factor made noisy deliberately

sta.ini - command file

sta.dat - noisy S(q) data

sta.gr - current reconstructed G(r) data ( during the calculations the constraint requiring that G(r) behaves as - 4prro has not been switched on at all)

ii) Percus-Yevick hard-sphere liquid structure factor with added systematic errors having a short period in q

sin.ini - command file

sin.dat - distorted S(q) data

sin.gr - current reconstructed G(r) data (during the calculations the constraint requiring that G(r) behaves as - 4prro has been temporarily switched on)

The original Percus-Yevick hard-sphere liquid structure factor, free of any systematic errors & noise, is also supplied. It is in file sniff.dat

- real experimental data for vitreous silica glass

si.ini - command file

si.iff - experimental S(q) data

si.gr - current reconstructed G(r) data (during the calculations the constraint requiring that G(r) behaves as - 4prro has been temporarily switched on)

We hope you will find IFO useful in your work.

Valeri Petkov and Radostin Danev

Sofia, January 1999, Bulgaria